Online adaption approaches for intensity modulated proton therapy for head and neck patients based on cone beam CTs and Monte Carlo simulations

Introduction

Head and neck (H&N) tumors are often very close to organs at risk (OARs) such as the parotids, larynx, submandibular glands or others. Due to this, H&N cases may benefit from steep dose gradients in the planned dose distribution to better spare the OARs while giving therapeutic doses to the target. Intensity modulated proton therapy (IMPT) is capable of producing such steep gradients and is an interesting treatment modality for these cases, offering potential benefits over photon treatments [Lee 2018, waddlw 2018]. However, proton plans, and in particular IMPT plans, can be very sensitive to uncertainties that may distort the planned dose distribution impacting the treatment quality [Ahn, Paganetti ...]. There are several sources of uncertainties, from which the patient setup and anatomy are two of the sources. The setup uncertainty arises from the patient location on the treatment couch being randomly different to the one recorded in the planning CT. The anatomy uncertainty arises from the patient anatomy evolution throughout the course of treatment, not only due to loss of weight modifying subcutaneous tissue and/or structure locations, but also due to posture on the treatment couch. In some regards it could be considered random and systematic, but the bottom line is that it is not easy to account for. Both uncertainty sources might modify the radiological pathway, by introducing high/low density materials in the beamlets path, affecting the planned dose distribution and impacting the planned target coverage and OAR sparing.

These uncertainty sources are usually taken into account during the planning procedure with the creation of safety margins around the target in an attempt to guarantee treatment quality throughout the fractions. As a consequence, the high dose volume in the patient is enlarged and organs at risk (OARs) receive higher doses. It is therefore desirable to shrink the safety margin to improve IMPT plan quality. An alternative to the target expansion is to perform robust IMPT optimization, which entails optimizing the plan considering different patient scenarios at the same time. With this approach, the resultant plan would provide proper target coverage and OAR sparing at delivery, as long as the current patient characteristics were included as a scenario during the optimization. This technique is able to maintain the original plan standards better than the safety margin approach in the general case. However, robust planning usually softens dose gradients, which, again, results in enlarged high dose volumes and higher dose to OARs compared to a plan performed in a single scenario. The previous techniques can be applied after careful selection of the beam angles to reduce the impact of uncertainties, which may give satisfactory results, but does not offer a general solution to account for setup and anatomy uncertainties and shrink safety margins at the same time.

An ideal solution that might fulfill both requirements is to perform online IMPT plan adaptation at every fraction. This is an ambitious interpretation of the role of online adaptation. A more modest interpretation of the application of online plan adaptation is to avoid treatment replanning in scenarios when the robust IMPT plan does not fulfill treatment standards. In both cases, online plan adaptation would have a big impact on the treatment quality through better plan quality or avoiding replanning that may delay the delivery of the full treatment.

The goal of online plan adaptation is to tailor an original IMPT plan to the setup and anatomy of each patient at each fraction. In other words, an online plan adaptation algorithm would tune a plan to a patient geometry different from where it was originally designed. The algorithm would need two inputs containing the new information: an imaging system capable of providing the patient anatomy at treatment position and an updated set of contours localizing the target and OARs. The algorithm would then generate an appropriate set of new or modified beamlets for the new geometry. All these steps should occur in a few minutes to fit clinical requirements. There are currantly three main options to provide the patient anatomy information: performing a new CT in the treatment room, using the information of a cone beam CT (CBCT) to deform the original CT or using a CBCT itself as the patient geometry [REFS, REFS, REFS]. However, an adaptation algorithm should be agnostic of the image source.

Several studies have been performed on online and offline IMPT adaptation. Automatic offline full plan reoptimization for H&N cases on deformed planning CT and contours was reported by [KURZ 2016], where new spot maps were created and optimized. Jagt et al. [JAGT 2017] published an online dose restoration procedure that is capable of restoring the daily dose distribution to the original plan in prostate cases on the original contours, correcting the spot energies and fully reoptimizing the spot weights. Recently, Bernatowicz et al. [BERNATOWICZ 2018] extended the work presented by Jagt et at. to allow other reoptimization strategies, applying them to nasopharynx, H&N and lung (1 case each). To the best knowledge of the authors, all previous approaches rely on full plan reoptimization, which, in turn, requires the costly calculation of full dose-influence matrices. A different approach without full plan reoptimization was presented by Moriya et at. [MORIYA] in a different context. They adapted the range shifter thickness of passive scattering plans for lung cancer cases. This methodology could be expected to have limited success if non-uniform energy shifts are needed, but it is an example of possible adaptation approaches that do not require full plan reoptimization. As summary, the publications here cited are either not an online algorithm (Kurz et al.) or they were not demonstrated on deformed contours (Jagt et al. and Bernatowicz et al.). Additionally, none of them include Monte Carlo simulations (MC) as dose calculation engine.

Another important uncertainty source in proton therapy comes from the dose calculation algorithm. In the case of H&N and other sites [GRASSBERGER lung and others], analytical dose calculations accuracy of proton beams might be compromised by tissue heterogeneity [JAN and many others]. If uncertainty margins due to the dose calculation algorithm are intended to be reduced in H&N plans, MC-based adaptation is desirable. However, using MC calculations for online adaptation imposes big time constrains on the design and applicability of the adaptation framework, as they are slower than analytical algorithms. Employing graphics processing units (GPU) for MC simulations and novel adaptation strategies, could potentially allow online IMPT plan adaptation on clinically applicable times.

Therefore, it is the main goal of this manuscript to demonstrate and perform online adaptation of IMPT plans on weekly acquired cone beam CT (CBCT) scans of H&N patients, with automatically propagated deformed contours and using a GPU MC code, gPMC [XUN, NAN], as dose calculation engine. Such algorithm would be applied while the patient is on the couch and could potentially allow security margins reduction or number of replanning steps minimization, increasing workflow efficiency. An immediate challenge of this goal was to design an algorithm that minimized the calculation burden to allow its online application. As a secondary goal, different adaptation approaches were studied. Some plan adaptation procedures may modify individual spot’s energy, increasing the number of energy layers and possibly rendering the new plan undeliverable. Other approaches could be successful, yet simpler than using the delivery system to modify the energy and position of individual spots independently. Therefore, the utilization of range shifters and couch shifts was compared against the independent adaptation of individual spot’s energies and positions.

Methods

At the beginning of this section an hypothetical adaptive delivery workflow is introduced and then the adaptation algorithm inputs are explained, setting the scenario in which this algorithm would be applicable. The adaptation algorithm is then detailed. Afterwards, the patient cohort is included. Finally, the approaches studied with the algorithm operation modes are explained along with the quantities selected to judge the performance.

Adaptation workflow

The presented algorithm aims to adapt IMPT plans online, before each fraction delivery, to account for patient setup and geometry uncertainties using GPU MC simulations. The current planning-delivery workflow, shown in figure 1 from the planning stage to the left, contains the uncertainties of the patient positioning and anatomy. This may be solved by employing an adaptive workflow, shown in figure 1 from the planning stage to the right.

Figure 1: The usual (to the left) and adaptive (to the right) planning-delivery workflows

The first step of such adaptive workflow would be to acquire information about the patient geometry, including a possible automatic generation of contours. Then the original IMPT would be evaluated on the new patient geometry with a GPU MC simulation. If the clinical requirements were still met, then the plan would be delivered. However, if the clinical requirements were not met, the patient information would be fed as inputs to the presented online adaptation algorithm. If the adaptation was successful, the patient would be treated, else, a replan would be necessary. It was an important goal of this manuscript to maintain high dose calculation accuracy at every step by using GPU MC simulations, therefore, gPMC was employed as dose calculation engine at the initial IMPT planning step, at the plan evaluation on the daily patient geometry and within the adaptation algorithm.

Inputs: CBCT, vector field map and contours

The required adaptation inputs are: the patient geometry contained on a CBCT, a vector field (VF) mapping the original planning CT to the CBCT, and a set of new contours localizing the structures in the CBCT.

CBCTs are known to provide limited accuracy for dose calculation [REFERENCES]. In order to employ the CBCT for dose calculation, an a priori planning CT-based scatter-correction algorithm [PARK, KURZ 2015] was applied. Park et al. have reported average accumulated WEPL errors across different locations of 2.3 ± 1.9 %. The CT and CBCTs were aligned at the plan isocenter, which better maintains the target coverage of the original plan quality at every evaluation, as opposed to aligning to a given OAR. This is not necessarily always the optimal clinical practice for every case if extra protection to a given OAR is desired, but the algorithm should work if this were the case.

The contours defined in the planning CT were propagated onto the weekly CBCT applying a VF. The VF map was calculated between the planning CT and the CBCT with deformable image registration (DIR) using the GPU parallelized B-spline algorithm in Plastimatch, an open source suite for radiotherapy and medical imaging [SHACKLEFORD, PLASTIMATCH]. The correctness of the structures on the weekly geometry was visually verified. To the best knowledge of the authors, there is no automatic and reliable procedure to generate the structure contours for H&N cases [REFERENCE???].

It is important to note that the accuracy of the CBCTs and the VFs employed in this manuscript does not affect the evaluation of the adaptation algorithm. It is understood that the CBCTs and VFs introduce uncertainties. However, even though the CBCTs and VFs may not perfectly represent the current patient geometry and deformation, they represent plausible geometries and deformations of the patient. As long as the geometry contained in the CBCTs is plausible, the algorithm should be able to adapt the initial plan to it. The same argument can be applied to the VF and, therefore to the propagated contours. Improvement of CBCT accuracy and automatic propagation/generation of structure sets is still a research focus in the field [CITATIONS]. Several approaches have been followed regarding the imaging modality best suited to perform online adaptation. An alternative option as the one here utilized is to register and warp the planning CT to the CBCTs. However, this method, for instance employed by Kurz et al [MORE EXAMPLES], has the issue that it has the accuracy of the DIR vector field. Another possibility is to have the availability of in-room CT, which may provide high quality images, but the positioning system might introduce uncertainties and it may not be as readily available as the CBCT.  All three options have drawbacks and advantages, but for the scope of this manuscript, the usage of scatter corrected CBCTs provides a the required scenario to demonstrate the algorithm.

Adaptation algorithm

The adaptation algorithm was designed as a two-step process, where, firstly, the beamlets positions and energies were adjusted to form the geometrical adaptation and, secondly, the beamlets weights were tuned, if found necessary by a GPU MC simulation.

After verifying with gPMC on the CBCT that the original IMPT plan needed adaptation (refer to figure 1), the algorithm would perform the next steps:

- Geometrical adaptation

- Simulation on CBCT

- Requirements not met: spot weights tuning

- Requirements not met: replan

- Geometrical adaptation

The geometrical adaptation adjusted the position and energy of the beamlets to the new geometry.

The geometrical adaptation steps are depicted in figure 2. Each spot in the plan was raytraced along its central axis until the end of range in the planning CT (1). The locations resulting from the raytracing were called endpoints. Due to the heterogeneity of the patient’s tissue and the subsequent deformation of the Bragg peak, the density of each voxel along the central axis was averaged with its surroundings. The average was calculated adding a set of 8 probes defined at the orthogonal plane around the central axis. The distance and relative weights between the probes and the central axis were calculated from the expected beamlet profile in water at the equivalent radiological depth. The angular position of the first probe was randomized. Because the patient deformation is captured in the VF, the set of original endpoints defined in the CT was warped applying the VF, yielding the position that should be occupied in the new geometry by each endpoint, and, therefore, by the high dose region of each beamlet (2). This was the same deformation previously applied to the contours propagation. The new position of each warped beamlet in the particle source plane was calculated next (3). Then, the shifted spots were raytraced in the CBCT applying the averaging algorithm (4). Finally, the energy was adjusted to match the end of range in the CBCT with the warped endpoint (5).

Figure 2: Geometrical adaptation steps. See text for explanation.

After the geometrical adaptation, the adjusted plan was simulated with gPMC on the CBCT. If the dose distribution fulfills the clinical requirements, the adaptation process would end and the plan would be delivered. If not, the spots weights would be tuned.

- Weight tuning

The usual IMPT problem solves an expression of the form of equation 1, with some objectives and constraints to the final dose distribution d. In equation 1, D is the full dose-influence matrix containing the dose distribution given at each voxel per beamlet and unit of fluence and ω is the beamlet weights to be calculated. However, calculating a full D matrix with GPU MC to solve such a problem is computationally demanding and not suitable for an online application. In order to comply with the requirements of an online application, a solution to extract enough information to define a D matrix from the geometrically adapted plan validation was found. It was observed that a minority of the spots in an IMPT plan carry the majority of the weight and, therefore, deliver the majority of the dose to the patient. This observation has a special meaning in the context of MC simulations, as the simulation uncertainty decreases with the number of simulated protons. As a consequence, this implies that there is a subset of beamlets simulated at the verification step with enough precision to be used for the weight tuning. Additionally, this subset is responsible for the majority of the dose in the patient and, therefore, may have the biggest influence on the plan performance.

1Dω=d2ω0Dω=d

To implement the ideas from the previous paragraph, the dose distribution calculated as verification after the geometrical adaptation was stored spot-by-spot, forming a weighted full D matrix (ω0FDF, where ω0 are the original IMPT weights and F stands for full). Accumulating the dose of each spot, the total dose per voxel given by the geometrically adapted plan was retrieved. The smallest set of spots carrying at least 50% of the total weight were extracted from ω0FDF, obtaining the matrix representing the subset ω0D. The number of selected spots was enforced to be at least 10% of the total to provide more flexibility to the weight tuning process. These two parameters, the weight and minimum number of spots percentages, could be tailored to each case and more advanced criteria could be developed in the future, however, provisionally, they were chosen empirically. The dose provided by the non-selected spots was accumulated as baseline dose. The prescription dose in the target minus the baseline was the dose that the selected spots should provide. The weights of the selected spots were then tuned to complement the baseline and provide coverage, while limiting hot spots in the target and sparing the OARs. The IMPT problem in equation (1) was therefore changed to equation (2), where ω are in practice scaling factors of the initial plan weights.

The same optimizer, Opt4D, from the original IMPT plan was used (see patient cohort section for details). The same set of OAR objectives and/or constraints can be enforced. If specific dose-volume histogram (DVH) points were utilized, the non-zero baseline should be taken into account, but this was not the case for the plans here included. In the case of the target, the weight tuning can be enforced to keep the same dose levels providing voxel-dependent maps to define the required dose per voxel (the baseline is voxel-dependent). After the weight tuning, another verification with GPU MC would not be required, as the new plan dose was computed from ω0Dω and the baseline.

Additionally, D matrices can be large files that could cause an overflow in the memory of GPU devices during MC simulations. To facilitate storage and speed up the optimization, D matrices are usually post-processed and voxels below a certain dose threshold are dropped, however, this approach cannot be used while D is being constructed. To solve this issue, gPMC was extended to score dose in arbitrary regions defined by input masks, allowing a significant reduction of the D matrices size. Voxelized masks representing the dose-bath were passed to the device, defining sensitive regions.

The algorithm was written in C++/OpenMP with the raytracing parallelized for NVIDIA GPUs with CUDA. The GPU MC code, gPMC, and the IMPT optimizer, Opt4D, were spawned as subprocesses, automatically providing the required inputs.

Patient cohort and original plans

A representative set of 10 H&N patients with 5-7 weekly CBCT each was studied, for a total of 60 cases. The set is summarized in table 1. All patients were treated with photons (IMRT), so new plans were created for IMPT. The plans consisted of 60 Gy(RBE) to the clinical target volume (CTV) with no safety margin delivered in 30 fractions. This provides a common and sensitive scenario to test the adaptation algorithm. A single dose level was selected in the CTV, creating a more challenging scenario for the plan adaptation, as allowing higher dose in the innermost part of the CTV potentially reduces the cold spots in the periphery of the structure. The field of view of the weekly CBCT images set a limit on the tumor extensions that was included in this study. We believe, however, that the cohort was representative.

The tumor locations include larynx, oropharynx, mouth, tonsil, tongue and hypopharynx. Tumor volumes ranged from 9.0 to 116.5 cm3, with an average of 53.2 ± 38.4 cm3. The target volume was observed to evolve throughout the CBCT sets. This evolution was a direct consequence of the deformation registered in the VF. In H&N cases, the target volume is often very hard to delineate and, therefore, the CTV volume change was not confirmed at every fraction. For the purpose of this study, the VF and the warped contours were accepted as long as the VF was observed to map the discernible structures well from the planning CT to the CBCTs. Additionally, the VF was required to present a smooth behavior around the non-discernible structures in terms of its nominal value and the Jacobian. The dice similarity coefficient, measuring the overlap between the original target contour and the contour propagated to each CBCT, was on average 0.83 ±0.09. The small dice coefficient value in patient 1 was due to the fact that this oropharynx case had a thin target and a small displacement caused a drop in this coefficient.

| Pat. No. | Tumor location | N fields | N CBCTs | Plan CTV volume (cm3) | Average CTV vol. ratio change (min, max) | Average CTV dice (min, max) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Oropharynx | 4 | 6 | 22.3 | 1.00 (0.97, 1.05) | 0.58 (0.50, 0.67) |
| 2 | Tonsil | 2 | 6 | 9.0 | 1.02 (0.94, 1.12) | 0.87 (0.83, 0.90) |
| 3 | Oropharynx | 3 | 7 | 30.7 | 0.93 (0.90, 1.00) | 0.82 (0.77, 0.88) |
| 4 |  | 4 | 6 | 81.3 | 1.03 (0.98, 1.06) | 0.79 (0.75, 0.84) |
| 5 | Hypopharynx | 3 | 5 | 59.6 | 0.97 (0.95, 0.98) | 0.89 (0.87, 0.91) |
| 6 | Mouth | 3 | 7 | 116.5 | 0.78 (0.75, 0.82) | 0.87 (0.83, 0.90) |
| 7 | Larynx | 3 | 6 | 25.0 | 1.21 (1.08, 1.34) | 0.84 (0.77, 0.88) |
| 8 | Tongue | 4 | 5 | 79.9 | 1.06 (1.04, 1.11) | 0.87 (0.82, 0.91) |
| 9 | Tonsil | 2 | 6 | 12.0 | 0.98 (0.95, 1.00) | 0.87 (0.83, 0.93) |
| 10 | Oropharynx | 3 | 7 | 95.9 | 0.96 (0.91, 1.02) | 0.89 (0.85, 0.92) |
| Summary: | - | - | 60 | 53.2±38.4 | 0.99±0.11 | 0.83±0.09 |

Table 1:  Patient cohort.

As stated at the beginning of the methods section, gPMC was also utilized as dose calculation engine to create the original IMPT plans. This approach increased the accuracy at this step and provides a consistent framework throughout the experiment, avoiding systematic differences between the adapted plan evaluations with gPMC and a treatment planning system (TPS). To create the IMPT plans, all the necessary information was extracted from the TPS, including the spot maps. D matrices were created with gPMC and optimized with Opt4D, an in-house optimization tool originally developed for temporo-spatial studies (TROFIMOV, 2005). Beam angles were selected to spare OARs and avoid artifact regions in the CT and the set of CBCTs.

Studied strategies and evaluation metrics

The geometrical adaptation and geometrical+weights adaptation approaches were individually studied. Four types of geometrical adaptation were implemented: 1) allowing free beamlet position and energy changes from the delivery system; 2) changing the isocenter with a couch shift; 3) changing the energy with range shifters; 4) constraining the position and energy changes with a isocenter and range shift. In total, 8 adaptation approaches were included in this manuscript. The isocenter shift was calculated from the average of the VF at the spots endpoints. The final compared distributions are listed in the following lines along with their identification keyword:

- None: Original IMPT plan delivered to the fraction geometry.

- Geo: Geometrical only adaptation with the 4 modes here described.

- Weights: Weight tuning after the 4 geometrical adaptation modes.

The modes are repeated here for clarity:

- Free: Free movement and energy correction of spots at the delivery system.

- Iso: Spots movement constrained to a couch shift.

- RS: Spots energy correction constrained to a range shifter.

- Iso-RS: Spots movement and energy correction constrained to a couch shift and a range shifter.

Dosimetric comparisons in the target were performed based on V95, V98, V107, V110 and others. The dose to OARs were analyzed in terms of mean and maximum dose. The performance was analyzed based on the cumulative dose at the end of treatment (cum.) and also on a weekly basis, extrapolating the values per week to full treatment for easier comparison. Although the cumulative dose is more clinically meaningful, if hypofractionation schemes are intended to be employed, good performance at every fraction would be desired. A constant RBE = 1.1 was applied throughout the study.

Results and discussion

In this section, the performance of the original plans on the weekly geometries was analyzed. Afterwards, the adaptation performance was investigated and compared, first using only the geometrical adaptation and secondly the weight tuning. Lastly, the time it takes for the adaptations was given. Appendix 1 in the supporting documents shows several examples of the VFs as probed by the adaptation procedure and also how the spot’s energy, positions and weights change with the adaptation procedure.

Original plans on weekly geometries

The performance of the original plans on the new geometries sets the scenario the adaptation algorithms must solve. This scenario was expected to be worse than most clinical cases because the plans were performed without CTV margins. However, it was the goal of this study to test the possibility of planning with no security margin for the anatomy and setup uncertainties.

Figure 3(left) shows the cumulative (cum.) and weekly V95unadapt−V95plan in the CTV per patient. It also shows the weekly variations of the same magnitude. The patient results are also aggregated with a boxplot to better represent the plan evolution trend. A gradual descend on the value of this parameter was observed as treatment progressed from week 1 to week 7. All the patients except 2 and 5 presented a 5% drop in at least one week. Half the patients showed a drop of the accumulated dose of at least 5% in V95, which was 100% in the plan. If V95 = 95 % were employed as replanning criteria for the target coverage, half the cases here presented should have been replanned from this criteria only. Few cases had a V100 > 90% and, although not shown explicitly for brevity, only patient 5 presented an accumulated V100 > 90 %, which, again, might trigger replanning for all cases, depending on the institution protocol. The average D98, mean and D2 dose percentage in the target changed from 98.9, 103.3 and 106.9 % at the original plans to 90.2, 101.8 and 108.4 % in the cumulative distribution, after being fully delivered to the patients.

Some cases, such as patient 5, did not show big CTV coverage drops (figure 3(top-right)), but the dose levels to OARs increased due to the geometry changes and the loss of conformality. On the other hand, patients such as 4, 7 and 8 showed drastic plan quality drops. The original plan and cumulative dose DVHs for patient 7 are shown in figure 3(right-bottom).

| Figure 3: Performance of the original plans on the weekly CBCTs. Top: Evolution of V95 at every week minus the original plan’s V95. V95 of the cumulative dose was also included. Individual patients are plotted as well as the total behavior (boxplots) Top-right: cumulative dose DVHs of patient 5. Bottom-right: cumulative dose DVHs of patient 7, showing big changes. |  |
| --- | --- |

In general, the studied conditions produced a decrease in parameters measuring target coverage with respect to the original plan, while parameters measuring target overdose increased, smoothing the target DVH. It is important to note that the parameters measuring target overdose should be judge having the target coverage in mind, as a D2 value similar or better than the plan only is relevant if the target is covered at the same time. Dose to OARs was also increased on average. The evolution of more CTV DVH points were included in figure 4 and are explained in the next subsection.

Geometrical adaptations

As a summary of the results here presented, the geometrical adaptation without weight tuning did not provide a general improvement upon the unadapted plan delivered to the weekly patient geometries. In other words, should the patient require replanning, this adaptation method would not avoid it.

Figure 4 shows boxplots of the distributions of V95, V98, V107, V110, D2 and D98 (%) containing the 10 patients. The lower/upper hinges correspond to the first and third quartiles (the 25th and 75th percentiles), the lower/upper whisker extends from the hinge to the smallest/largest value no further than 1.5 times the inter-quartile range from the hinge, observations beyond the whiskers are plotted individually as outliers. Boxplots for the original plans and the plan accumulated after being delivered to the weekly geometries with and without geometrical adaptation are shown and separated by color, allowing the comparison of several CTV DVH bands. Each boxplot represents the given parameter corresponding to the cumulative dose distribution delivered to the weekly anatomies for the 10 patients, except for the plan boxplot, which acts as the reference baseline. Boxplots are included for the original plans in the planning CTs (Plan), the plans delivered to the weekly geometries with no adaptation (None) and the 4 geometrical adaptation modes described in the methods: free energy and position changes (Free), isocenter shifts (Iso), range shifters (RS) and the last two combined (Iso-RS).

The plot shows the original plan delivered 95 and 98 % of the prescription dose to the whole CTV volume (orange, leftmost boxplots). The volume percentage decreased as the dose increased until it was ~5% for V107 and close to 0% at V110. The average D98, D2 doses were 99.0 and 107.0 % for the original plans. However, this behavior was not reproduced when the original plans were delivered to the weekly geometries without adaptation (None method, yellow, second boxplot from the left), as explained in the previous subsection.

Figure 4: Performance of the geometrical-only adaptation modes compared to the original plan and the original plan accumulated after delivering to the weekly CBCTs. The geometrical-only adaptation fail to reproduce the original plan’s values.

It is clear from figure 4 that the geometrical adaptation methods failed to reproduce the original plans’ boxplots. The Free and Iso methods improved in some cases the performance with respect to not adapting the plan, according to V95 and V98, but in others they performed worse. The spots movement created hot-spots in the CTV as show by V107, V110 and D2. Both methods adapting with range shifters (RS, Iso-RS) worsed the CTV coverage significantly.

In general, as stated in the first sentences of this subsection, the geometrical adaptations without weight tuning did not recover good plan quality. There are several reasons that might account for this issue, individually or in combination. Although theses methods relocate the spots’ end-of-range position in the target according to the deformation, the change of shape of the dose distribution of each spot after adaptation produced by the new patient geometry can generate cold/hot spots in the target that impact the plan quality. Not only the materials traversed may affect the adapted beamlets, but also the beamlet energy may have changed, which would change the beamlet width and energy spread coming from the delivery system. Additionally, the relative positions between endpoints may be different if the VF contains non-parallel deformations, creating potential cold/hot-spots. Because the VFs were probed at certain positions, there was an underlying assumption that the VFs behaved smoothly. Any non-smooth regions of the VFs might not be captured by the probing procedure, but they would certainly be used to warp the original contours, potentially creating deformations that the algorithm would not take into account. This last issue may not be likely to happen in a scenario where the new contours and the VFs are fully tested.

Geometrical adaptations with weight tuning

The weight tuning algorithm described in the methods, section 2.3.2, was applied after simulating the geometrical adaptations in a per-spot fashion.

As shown in figure 5, the weight tuning was able to recover good plan quality in terms of the CTV DVH parameters. Again, the boxplots represent the distribution of values for the given parameter evaluated at every patient’s cumulative dose distribution. The Free and Iso methods were seen to better represent the distribution of results shown by the initial plan boxplots in all dosimetric indices, except in V107, where the adaptation showed higher values. These higher values, were not reflected in V110, which means that the DVH had a high enough gradient at that region. Comparing against the non-adapted and the geometrical-only adaptations (shown in the previous figure), all modes represented an improvement. The methods utilizing range shifters necessitated higher dose levels to achieve similar coverage, as shown in V107, V110 and D2. This figure provides information about the general distribution of DVH parameters values in the same or similar scale as in figure 4, but not  about the patient-by-patient difference with respect to the original plan.

Figure 5:  Performance of the weight tuned adaptation modes compared to the original plan and the original plan accumulated after delivering to the weekly CBCTs.  The Free and Iso modes closely reproduce the original plan’s values. RS and Iso-RS present lower V95 and V98 and higher V107 and V110. The plot scale may have hidden the boxplot color, the order is the same as in the legend. Also, the plotting scale dose not allow easy comparison in all cases.

Figure 6 focuses on the patient specific differences with respect to the original plan (method – plan, Δ%). Weekly variations of the dose distributions were also included here. The methods utilizing range shifters were not included in this figure for clarity and because the previous plot showed they had worse performance than the Free and Iso methods. A mode detailed studied of the different modes was included in figure 8. Table 2 shows the results for the selected dosimetric indices for the original plan, its performance on the weekly CBCTs and the weight adaptation modes.

Figure 6:  Distribution of several parameters difference between the adaptations and the original plans. The difference was taken patient by patient, either using the cumulative dose distribution after adapting every fraction individually, or using the weekly dose distributions themselves. Both modes here included behaved similarly. Target coverage was well maintained in the cumulative distribution and the weekly distributions presented bigger fluctuations.

The distribution of differences from the cumulative doses shown in figure 6 are analyzed first. The difference between the original plan and the Free and Iso modes was small in the CTV. The Free mode showed slightly improved nominal values with respect to the Iso mode, but the difference was not statistically significant. The average difference across patients between the adaptations and the original plan for V95 was less than 1% as shown in the figure and in table 2 (99.4 – 99.9 for the free mode). For V98 it was less than 2% (97.4 – 98.9 for the free mode). In order to deliver such coverage, the adaptations increased V107 by ~ 4% on average, from 2.4 to 6.4 %, however it impacted V110 by less than 1%. The minimum and maximum dose were also well maintained with average differences smaller than 2%. These small differences contrast with the values shown by the original plan without the adaptation algorithm.

The weekly results showed a wider distribution than the cumulative doses. The results for the individual fractions tended to be worse than the cumulative results. Fluctuations occured at every fraction that canceled when summing the total dose. In this case the Free mode also performed better than the Iso, but, again, not significantly. The average V95 and V98 of the weekly adaptations in Free mode were 99.0 and 96.4, meaning 0.9 and 2.3 % drops from the original plan. The bigger impact of the weekly fluctuations were seen at V107, where differences of up to ~30% were observed, with an average increase from 2.4 to 12.8 between the plan and the Free mode.

|  |  | Cumulative |  |  |  |  | Weekly |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| All in % |  | Min | Mean ± sd | Median (IQR) | Max | Min |  | Mean ± sd | Median (IQR) | Max |
| V95 | Plan | 99.6 | 99.9 ± 0.1 | 99.9 (0.0) | 100.0 | 99.6 |  | 99.9 ± 0.1 | 99.9 (0.0) | 100.0 |
|  | Free | 96.7 | 99.4 ± 0.9 | 99.7 (0.2) | 99.9 | 96.1 |  | 99.0 ± 1.0 | 99.4 (0.6) | 100.0 |
|  | Iso | 96.8 | 99.1 ± 1.2 | 99.7 (0.4) | 100.0 | 94.9 |  | 98.9 ± 1.1 | 99.4 (0.7) | 99.9 |
|  | None | 84.0 | 94.1 ± 5.2 | 95.6 (7.3) | 99.4 | 79.2 |  | 93.9 ± 5.2 | 95.2 (8.0) | 100.0 |
| V98 | Plan | 97.2 | 98.9 ± 1.0 | 99.4 (1.4) | 100.0 | 97.2 |  | 98.9 ± 1.0 | 99.4 (1.4) | 100.0 |
|  | Free | 93.1 | 97.4 ± 1.7 | 98.2 (0.8) | 98.6 | 92.1 |  | 96.4 ± 1.8 | 97.2 (2.6) | 98.5 |
|  | Iso | 93.1 | 97.2 ± 2.0 | 98.0 (1.8) | 99.0 | 90.6 |  | 96.5 ± 1.9 | 97.4 (2.0) | 98.4 |
|  | None | 71.4 | 87.3 ± 9.4 | 89.2 (14.8) | 97.7 | 63.7 |  | 86.0 ± 9.0 | 87.5 (13.4) | 99.5 |
| V107 | Plan | 0.2 | 2.4 ± 1.9 | 2.4 (3.2) | 5.2 | 0.2 |  | 2.4 ± 1.9 | 2.4 (3.2) | 5.2 |
|  | Free | 0.5 | 6.4 ± 4.7 | 5.1 (3.7) | 14.5 | 5.2 |  | 12.8 ± 5.8 | 12.0 (8.4) | 30.6 |
|  | Iso | 0.4 | 6.6 ± 6.2 | 4.1 (7.5) | 19.2 | 4.6 |  | 12.3 ± 6.4 | 9.8 (9.9) | 31.7 |
|  | None | 0.1 | 6.3 ± 5.9 | 4.9 (6.2) | 19.5 | 0.8 |  | 9.0 ± 5.0 | 8.3 (5.3) | 26.1 |
| V110 | Plan | 0.0 | 0.1 ± 0.1 | 0.0 (0.0) | 0.3 | 0.0 |  | 0.1 ± 0.1 | 0.0 (0.0) | 0.3 |
|  | Free | 0.0 | 0.4 ± 0.7 | 0.1 (0.2) | 2.1 | 0.0 |  | 2.1 ± 2.4 | 1.1 (2.5) | 11.8 |
|  | Iso | 0.0 | 0.9 ± 1.9 | 0.0 (1.1) | 6.0 | 0.1 |  | 2.1 ± 3.1 | 0.6 (2.9) | 16.6 |
|  | None | 0.0 | 1.1 ± 1.3 | 0.6 (1.1) | 3.8 | 0.0 |  | 2.2 ± 2.4 | 1.8 (1.6) | 15.2 |
| D98 | Plan | 97.5 | 98.9 ± 1.0 | 99.1 (1.3) | 100.3 | 97.5 |  | 98.9 ± 1.0 | 99.1 (1.3) | 100.3 |
|  | Free | 93.5 | 97.5 ± 1.5 | 98.2 (0.8) | 98.5 | 91.2 |  | 96.7 ± 1.7 | 97.3 (1.3) | 98.3 |
|  | Iso | 92.7 | 97.2 ± 2.1 | 98.0 (1.3) | 99.0 | 86.5 |  | 96.6 ± 2.2 | 97.5 (1.5) | 98.3 |
|  | None | 67.5 | 90.2 ± 8.9 | 92.8 (6.8) | 97.8 | 40.5 |  | 89.5 ± 9.0 | 91.3 (8.8) | 99.0 |
| D2 | Plan | 105.8 | 106.9 ± 0.7 | 107.1 (1.0) | 108.0 | 105.8 |  | 106.9 ± 0.7 | 107.1 (1.0) | 108.0 |
|  | Free | 106.3 | 108.0 ± 1.1 | 107.9 (0.8) | 110.2 | 107.8 |  | 109.7 ± 1.4 | 109.3 (2.0) | 113.7 |
|  | Iso | 106.3 | 108.2 ± 1.8 | 107.6 (1.9) | 112.3 | 107.7 |  | 109.8 ± 2.4 | 108.8 (2.3) | 122.8 |
|  | None | 105.8 | 108.4 ± 1.7 | 108.8 (2.2) | 110.7 | 106.3 |  | 109.8 ± 1.9 | 109.8 (1.5) | 117.5 |

Table 2:  Dosimetric values from the cumulative and weekly dose distributions.

The difference in mean and max dose between the weight tuned adaptations minus the original plans is shown in figure 7 for all adaptation modes. The OAR dose levels were kept similar to the original plan by the Free and Iso modes, while the other modes displayed increased dose to the larynx and mandible. Only the larynx, mandible and submandible glands have significant dose levels across patients, so the analysis was focused on them.  Because the patient anatomy changed, in some situations the weight adaptation algorithm was able to find better sparing than the original plan in the original patient geometry, confirming that the original plan dose distribution might not the optimal for every geometry.

Figure 7: Dose difference given to OARs by the adaptations with respect to the plans. The dose levels were well maintained by the adaptations, specially the Free and Iso modes.

With the information so far provided, it has been shown that the weight tuning algorithm was able to maintain good CTV coverage while limiting the target and OAR overdose. The Free mode was seen to produce slightly better results than the Iso mode. Both modes kept target and OAR overdose to lower levels than the modes using range shifters. However, the adaptation modes have not been explicitly compared. In order to do that, the Free mode was taken as reference and dosimetric differences between the modes and the reference were calculated. The results were plotted in figure 8 for the CTV and main OARs.

Figure 8: Comparison of adaptation modes. The values show the difference between the given mode and the Free mode. The Iso mode was seen to perform similarly to the Free mode, but the Iso-RS and RS performed worse.

The modes using range shifters again delivered higher dose than the Free mode to the target and OARs. The Iso mode had a lower minimum dose in the CTV, which explained the lower V95 and V98 seen in table 2. The Iso mode also displayed lower dose to the larynx, but higher to the submandibular glands. However, the dose to OARs had a high dependency on the specific case, so no conclusion could be drawn for the small difference here observed. The comparisons performed on the weekly dose distributions showed similar behavior as the cumulatives, but with higher variance. In conclusion, the Free mode performed better than the modes with the rage shifters, as had already been indicated in the previous series of plots, and only slightly better than the Iso mode according to V95 and V98 in the CTV (table 2). The other parameters trying to assess the difference in performance between the Iso and the Free mode did not yield any statistically significant difference.

Figure 9 shows DVHs of four patients and the performance of the Free adaptation mode with weight tuning. The adaptation can be seen to restore a plan quality comparable to the original plan both in the CTV and in the OARs.

Figure 9: Patient DVHs of the original plan,  the plan unadapted and fully delivered to the weekly scans and the plan adapted with Free geometrical adaptation and weight tuning (solid line).

Time performance

The framework time is reported without considering the creation of the CBCT and the DIR.  It was summarized step-by-step in table 3. All the calculations were performed with a  XXXXXX.

The geometrical adaptation step includes the data transfer between the host and the GPU, the VF probing and geometry changes to adapt to the internal coordinates. It took an average of 16.9 seconds, however, with a dedicated system the overhead of operations would be minimized and the algorithm itself would be expected to run much faster. It is not clear either whether an CPU parallelization would be faster for this step, given that the number of parallel threads in the GPU is only the number of spots.

The simulation with gPMC was done in the mentioned GPU card and took between 115.6 and 419.2. These values can be drastically reduced by launching the calculation in a multi-GPU system. Newer GPUs and motherboard-GPU connections would also improve the performance, although it is hard to predict what the expected improvement would be. Just with a 10 GPU system of the device here utilized, the calculation time would be in the order of 10-60 s for any of the cases here included.

The weight tuning presented a wide range of calculation times, from 12 to 198 s. The calculation times of optimizations processes are hard to predict, however, it is proportional to the size of the problem. In this case, all dose calculations were perform on the CT grid. Should this grid be resampled to a coarser one, the optimization time would be expected to drop. Another factor impacting the weight tuning time is the number of spots to consider. Currently  the spots selection criteria does not take into account the position of the problematic areas. Including that information might reduce the number of spots to be considered, reducing the total tuning time. Lastly, the parallelization of the code would also be expected to have a big impact. A ballpark duration after reducing the dose calculation grid and the parallelization could be in the range of 5-120 s, however, this is speculative, which is the reason it was not included in the table.

| (All in s) | Minimum | Average | Maximum | Expected |
| --- | --- | --- | --- | --- |
| Geometrical adapt. | 11.7 | 16.9 | 26.6 | 1 - 5 |
| gPMC simulation | 115.6 | 261.9 | 419.2 | 10 - 60 |
| Weight tuning | 12.0 | 44.8 | 198.0 | ?? |
| Total | - | 322.7 | - | ~ 60 - 120 |

Table 3: Time taken with the available hardware and design and the expected time with the improvements explained in the text.

Conclusions and outlook

An algorithm was presented that was capable of yielding adapted plans of similar quality as the original treatment plans. Adaptation of spot’s position and energy (here termed as geometrical adaptation) was proven to be insufficient. However, tuning the weight of a subset of spots after this geometrical adaptation was proven successful. Several geometrical adaptation modes were individually tested and compared. It was shown that allowing the change of individual spots position and energy (the Free mode) yielded the best results, although the using an isocenter shift with free energy changes also yielded results very close to it. The usage of range shifters for the energy adaptation may prevent the creation of many individual energy layers, which would be undeliverable in some systems, but they gave higher doses to the CTV and OARs.

The timing here reported would not grant its usability within an online adaptation workflow, but simple parallelization and hardware improvements may bring the framework into clinically applicable times.

Thus, the algorithm here presented has the potential to be applied within an online adaptation workflow. Its application may allow further margin reduction and the consequent plan quality improvement. In order to do that, further investigations in the imaging, deformable image registration and automatic contour generation should be conducted. Of course, the presented algorithm could be improved in areas such as better spot selection for weight tuning, code parallelization and the creation of a fully integrated code.

APPENDIX 1